INTRODUCTION

Did you ever wonder how ISRO launched 104 satellites in one go? Are you curious about how Aeroplanes fly? Does seeing a drone fly drags you to a question-how is it so stable!!!? what is the science behind? These are the very questions where we started our journey at Drisht and came forward to study, implement, and share the knowledge about Aerodynamics and Aerospace. As we all know that aerospace is one of the rapidly growing sectors and countless researches are going on in space and rocket technology.

Team DRISHTI is also trying to learn and spread knowledge about Aerodynamics and Aerospace. We at Drishti started working on Quadcopter and RC Planes. Team Drishti is working on the development of a control system(Flight Controller) for Quadcopter. We have been working on RC Plane and had recorded number of successful flights. To test our knowledge and implementation, Team GARUDA, Drishti participated in Boeing Aeromodelling Competiton, TechFest IIT, Bombay, and qualified for Round 2. The final aim is to automate the RC Plane using our own control system. We also launched a Single Stage Solid Propellant (J Class Rocket Motor). Ornithocopter, Howercopter, and Magnus Plane were other projects we worked on.

RC PLANE

In today’s modern world Airplanes are nothing new to we humans, we have seen or may have experienced an airplane flying at some point of time in our life. But this idea of creating a mechanism to make humans fly had started rising into human brains many centuries ago. Many famous stories of the ancient times involve the idea flying, such as Greek legend of Icarus & Daedalus, and the famous Indian epic Ramayana involves a flying vehicle called “Vimana”. The study of Aerodynamics has taken a great leap and many new airplane models are continuously developing. The RC Planes were firstly introduced for research purposes, the miniature planes were first constructed as a study model of the airplanes under research to study the forces acting on the newly designed models and the design was edited to extract the desired result. The model is a real aircraft that flies and operates by the same principles as its full-scale counterpart and the only difference is size and weight.  Along with the research purpose specially designed RC planes are also used in gathering weather readings, surveillance, etc.

Overview of RC Plane: -

RC Planes consists of various components which can be broadly categorized in to mechanical system and Electronic System. The Mechanical system mainly contains the Fuselage, wings, Empennage and Control Surfaces. Fuselage is considered as the main part of the airplane. It is the main body section of the airplane. Basically, it is long hollow structure which holds all the parts of the airplane together. Wings are the most important part of the airplane. The wings are airfoils attached to each side of the fuselage and are the main lifting surfaces that support the airplane in flight. The whole assembly of the tail including horizontal stabilizer and  vertical stabilizer is known as empennage. Control Surfaces are mainly used to control and manoeuvre the aircraft. RC plane mainly contains three control surfaces i.e. Ailerons , Rudder and Elevator. Ailerons are the moving surfaces usually placed near the tips of the wings. The function of the aileron is simple, by moving it in upwards or downwards direction, It controls the roll motion of the plane. Horizontal stabilizer contains one movable part which helps in pitch moment known as elevator. Rudder is movable part of the vertical stabilizer and it helps to control the yaw motion of the aircraft. The electronic system consists of Motors and propellers which provide thrust to the aircraft , ESC which converts DC Power coming from the Battery to AC Power going to the motor, Battery, Reciever and Transmitter and  Servo motors which are used to actuate the motion of control surfaces,

Considering various design parameters of RC Plane, Team Drishti has designed and then perform various analysis over the design and after satisfactory result of analysis, we have manufactured various RC planes and also conducted various successful Flight tests.We have also manufactured various indoor testing instruments like CG Balance setup, Thrust setup and 3D analyser which would give the characteristic of the manufactured aircraft which are helpful while conducting the Flight Test. We have also participated in Boeing Aeromodelling competition at IIT Bombay and successfully got qualified for Round 2.

RC Plane has very wide range of possible applications and various design to be implemented for various specific applications. We have achieved a stable flight  and we are further planning to use flight controller specially designed for RC Plane and hence using telemetry we can get the coordinates, velocity, altitude and various other parameters. We are also planning of using GPs with which plane would travel from one location to other as per the input given and this travel would be autonomous and it would be after take-off and before landing.

ROCKET

Sounding rockets are one or two stage solid propellant rockets used for analyse the lower atmospheric regions as well as re-entry systems. The Sounding rockets are affordable systems to test prototypes of new components.

We have build Single Staged Solid Propellant (H, J Class Rocket Motor) Rocket. The main working component in the rocket is its motor. Other parts in the rocket are the body, nose, nozzle, and fins. We here make solid propellant rocket.

The rocket fuel mixture consists of Fuel and Oxidizer. There are number of fuel and oxidizer combinations which are categorized by Thrust, Size, Burn-time and Initial knock. We have used one of the most standard combination refered as Rocket candy or R-candy which is a mixture of sugar (Fuel) and Potassium Nitrate (Oxidizer).

The Rocket motor consists of Rocket fuel, Casing and Nozzle arrangement. The main aim is to achieve maximum height possible and for that initial thrust plays crucial role which can be done by minimizing the fuel burning time. Nozzle arrangement can be beneficial because of small orifice which allows the burning fuel to pass through a single path with high velocity which eventually decreases burning time

Bluff body is exposed to more drag. Drag will degrade the performance of rocket in terms of Achievable height and it also reduces the overall effective thrust. So in order to overcome this Nose is mounted at the top section of the rocket which reduces the drag and increases the performance because of aerodynamic shape.

#### MAGNUS EFFECT

The Magnus effect is an observable phenomenon that is commonly associated with a spinning object moving through the air or another fluid. The path of the spinning object is deflected in a manner that is not present when the object is not spinning.

Now there are many ways to generate lift using this theory.

#### DIFFERENT TYPE OF DESIGNS

LONG CYLINDER

In long cylinder Magnus effect can be seen very highly and the force generated is also high. Torque provided to long cylinder is relatively less

SPHERE

Sphere shape objects can also produce magnus effects ,like soccer balls , tennis balls etc . But it cannot produce a high lift, so not used in magnus plane

RECTANGULAR

It is similar to cylindrical but here a rectangular plate by rotating at high speed, it seems to be cylindrical , it requires more torque , less moment of inertia ,less lift.

COMBINATION OF CYLINDER AND AEROFOIL

It is combination of both cylindrical and normal aerofoil ,it generates very good lift ,drag will be more , very difficult to manufacture

Quadcopter

We developed Simulink based quadcopter simulation which  is intended to reduce cost and running a simulation will represent dynamic performance of your vehicle. The simulation is intended to act as a sort of starter kit to help accelerate you towards the goals of speeding up the process of system design and provide one with tools to explore quadcopter control design techniques. The simulation represents a synthesis of several author’s approaches to modeling the behavior of a quadcopter, with a little bit of additional modeling added. Many important effects (most obviously aerodynamic effects such as blade flapping) are ignored or dramatically simplified, so one will need to make sure to understand the limitations of the model in order to get reliable service out of it. Since it is written in Simulink and MATLAB code.